Daily living with the subretinal chip

Prof. Dr. Dr. h. c. mult. Eberhart Zrenner Center for Ophthalmology

Institute for Ophthalmic Research

University of Tuebingen

Tuebingen, Germany

Background: Since 2005 twenty nine patients have received a subretinal electronic implant in Tübingen, Germany, after 10 years of preclinical testing. Results from a pilot study obtained retinitis pigmentosa patients (n=11) with a wirebound implant were recently published in Proc. R. Soc. B 278, 1489–1497, 2011 (http://rspb.royalsocietypublishing.org/content/early/2010/11/01/rspb.2010.1747). The new implant version (IMS Alpha from Retina Implant AG, Reutlingen, Germany), applied in further 18 patients since 2010, is a wireless, completely subdermal implant with 1.500 light sensitive photodiodes, amplifiers and electrodes, placed behind the retina in the foveal region.

Methods: Each of the 1500 subfoveal photodiodes within a 11 by 11 deg field controls an amplifier that, depending on the strength of the light, ejects currents onto bipolar cells via an electrode (Stett et al. Vision Research, 2000). Power and control signals are supplied by inductivity via a subdermal retroauricular coil from which a subdermal cable leads to the eyeball.

Results:  All patients of the first monocentric phase of the trial in Tübingen were able to perform the function tests except one where a loss of inner retina function was observed after surgery. Results in all other patients  were: light perception: 9/9, light localization: 8/9; motion recognition 5/9; grating resolution 8/9 (up to 3,3 cycles/degree); Landolt C rings 2/9 (up to Visual Acuity = 0,036); recognition of geometric objects 8/9; recognition of objects in table setup 8/9; Letter reading 4/9; clock hands reading 3/9; grey scale differentiation 6/9; improved outdoor mobility and activity 5/9.

Patients reported numerous beneficial visual experiences in daily life with regained recognition of unknown objects, recognition of facial or clothes’ characteristics, moving objects in nature and traffic, improved self-sustaining actions. Examples: Near vision: Localizing plates, cutlery and tableware in 4AFC mode, recognizing plates and their content and fruits in restaurant, facial characteristics of persons, items on working desk. Far vision: Recognizing clouds; identifying the borders of cars by metallic reflections or walking along a white line; orientation along street lamps. Spatial resolution: Resolving the position of clock hands, stripe patterns up to approx. 0.33 – 0.44 cpd, reading letters 4 to 8 cm high, localization and recognition of square, round or rectangular objects presented at visual angles of 6° to 8°. Motion: Seeing hand moving, recognizing walking direction of a white goose, circumvention of obstacles, seeing car lights approaching. Eye-hand coordination: Pointing at objects and moving them, grasping objects.

Conclusions: Careful psychophysical testing and patient daily-life reports show that the wireless Alpha-IMS implant restores useful visual abilities in blind RP-patients with visual acuity up to 20/550 (Landolt-C-ring). There was a high variability of functional outcome among patients, mainly depending on the particular form and stage of retinal disease. Interestingly the learning period was quite short; most of the regained visual abilities were achieved within the first two weeks. Special attention is paid to the fact that microsaccades of the eye are important for constantly refreshing the image. Subretinal surgery for positioning chips subfoveally is safe and the multicenter part of the study has been started in Oxford, London, Hongkong and Budapest.

Author Financial Disclosure Information: Eberhart Zrenner, Support by Retina Implant AG.